# SpectraForge: A Physics-Grounded Synthetic Benchmark for Band Selection in Multi-Excitation Hyperspectral Imaging, and an Honest Appraisal of Autoencoder-Based Selectors

Author: Narek (with the SpectraForge research agent)

Date: June 2026

# Abstract

We present SpectraForge, a physics-grounded synthetic generator for multi-excitation hyperspectral

imaging (ME-HSI), and a rigorous, significance-tested study of band-selection methods built on it — in

particular the autoencoder (AE) + latent-perturbation idea and its convolutional variant (CAE). The

generator renders excitation–emission cubes from a parametric fluorophore model obeying the standard

laws of fluorescence photophysics (Kasha, Vavilov, Franck–Condon, Stokes shift), with optional

inner-filter, reabsorption, Förster resonance energy transfer (FRET), scatter, photon/read noise, and

fixed-pattern clutter. Crucially, scenes are constructed so that **variance is decoupled from

informativeness, making the benchmark an honest test. Across ~32 experiments in three rounds**, with

full freedom over both the generator and the models, we find: (i) once debugged, the AE+perturbation

method becomes competitive with classical baselines but **does not beat a random-selection control under

realistic clutter, and never beats using the full spectrum**; (ii) the value of selection is

compression, and where it matters it requires labels (supervised mutual information / RF-importance);

(iii) the AE provides no distinct advantage over PCA in any per-pixel role (selection or denoising);

and (iv) the one structural win is that a convolutional/spatial model is *necessary* — and

per-pixel methods *provably blind* — when discrimination is carried by spatial texture. We document the

generator in full detail and provide an application guide so the benchmark is fully reproducible.

# 1. Introduction

A multi-excitation hyperspectral cube records, for every pixel, an excitation–emission matrix (EEM):

the sample is illuminated at several excitation wavelengths and, for each, the emission spectrum is

acquired across many bands. The per-pixel feature vector (excitations × emission bands) is high

dimensional and largely redundant; only a small subset of (excitation, emission) bands carries the

information that distinguishes materials. Band selection chooses that subset, so a cheaper instrument

can acquire only the informative bands or a downstream classifier can run on a compact feature set.

The idea under test is AE + latent-perturbation selection: train an autoencoder to reconstruct the

cube; perturb each latent dimension and measure how strongly each input band's reconstruction responds;

accumulate these per-band influences and select the top-k. It is attractive because it is *unsupervised*

yet could in principle capture *nonlinear* structure linear methods (PCA loadings, variance) cannot.

This paper documents (a) the physics-grounded generator that makes the test honest and (b) the

significance-tested verdict on whether the AE/CAE actually helps.

# 2. Synthetic data generation: the SpectraForge forward model

## 2.1 The laws of fluorescence photophysics

The generator is grounded in the real photophysics of fluorescence so that synthetic EEMs are

structurally faithful and "informative" means what it means physically:

Jablonski / electronic states. A fluorophore absorbs a photon (S₀→S₁/S₂…), relaxes

non-radiatively to the lowest vibrational level of S₁ (internal conversion + vibrational relaxation,

picoseconds), then emits.

Kasha's rule. Emission occurs from the lowest vibrational level of S₁ regardless of excitation —

so the emission shape is independent of excitation wavelength, and the EEM is approximately

trilinear (excitation profile × emission profile × concentration). This is the basis of PARAFAC.

Vavilov's rule. Quantum yield is (to first order) independent of excitation wavelength; only the

amount absorbed changes with excitation.

Franck–Condon principle. Vibronic band intensities follow Franck–Condon factors, giving the

broadened, roughly mirror-image excitation/emission envelopes; the optional vibronic shoulder models

the progression that gives a dye its characteristic *shape*.

Stokes shift. Emission is red-shifted from absorption because energy is lost to vibrational

relaxation before emission.

## 2.2 The parametric fluorophore model

Each fluorophore is a frozen dataclass with Gaussian excitation/emission bands (dilute-regime model):

its dilute contribution is `extinction · quantum_yield · excitation(λ_ex) · emission(λ_em) ·

concentration`. The excitation profile is a peak-normalised Gaussian; the emission profile is an

area-normalised (split-)Gaussian with optional em_skew (red tail) and a vibronic Franck–Condon

shoulder ~1300 cm⁻¹ to the red. Figure 1 shows the discriminative dyes (modest brightness, overlapping

emission, distinct excitation) and the bright nuisance autofluorophores.

Figure 1. Parametric fluorophore spectra. Left: three discriminative dyes (dashed = excitation, solid = emission) — modest brightness, overlapping emission, distinct excitation peaks. Right: two bright nuisance autofluorophores (NADH, lipofuscin) that dominate band variance but carry no class information.

## 2.3 The forward model

For each excitation wavelength λ_ex, the renderer computes a scaled lamp/exposure/power factor and sums

each fluorophore's contribution `(concentration · extinction · quantum_yield · excitation(λ_ex)) ·

emission(λ_em)` into the cube, accumulating per-pixel excitation absorbance for the inner-filter option.

With artifacts and physics off, the model is exactly linear: render(A+B) = render(A) + render(B).

## 2.4 Scene generation: variance ≠ informativeness

The single most important design choice. A confounded scene paints (i) discriminative dyes with

smooth random concentration fields whose per-pixel argmax defines the class label — these are dim

and/or spectrally overlapping (low-variance, discrimination in shape); and (ii) nuisance dyes with

their own independent fields — bright, high spatial variance, but class-irrelevant. A turbidity field

drives spatially-varying scatter. Consequently the highest-variance bands carry no class information.

Figure 2 shows class labels, example bands (a discriminative dye band, a Rayleigh-scatter band, a

nuisance band), the mean emission per excitation, and the variance-vs-informativeness scatter whose

near-zero/negative correlation is the property that makes the benchmark honest.

Figure 2. The forward model and the variance≠informativeness property. Top: class labels and example rendered bands (discriminative, Rayleigh scatter, nuisance). Bottom: mean emission per excitation; the per-band variance vs per-band discriminability scatter (corr printed) showing the highest-variance bands are NOT the informative ones; an example rendered band.

## 2.5 Artifacts: scatter and noise

On top of the clean cube the generator adds Rayleigh scattering (elastic, at λ_em = λ_ex and its

second order at 2λ_ex), Raman scattering (inelastic, fixed energy offset), each scaled by the

per-pixel turbidity field; then photon (shot) noise (Poisson via photon_scale) and Gaussian

read noise (read_sigma). Scatter ridges are high-variance and carry zero chemical information — a

deliberate trap for variance-based selectors.

## 2.6 Optional nonlinear physics and instrument clutter

Four options make the data progressively more realistic and nonlinear:

Primary inner-filter (Beer–Lambert). Local signal attenuated by exp(-s · A_ex), a per-pixel

scalar — the first deliberate nonlinearity in concentration.

Reabsorption / secondary inner-filter. Emitted light reabsorbed by exp(-s · A_em(λ)), where the

absorbance is per-emission-wavelength; because absorption overlaps the blue edge of emission, this

reshapes the band (suppresses the blue edge, apparent red-shift) as a function of concentration —

a nonlinearity that relocates information into band shape (Figure 3).

FRET. For donor→acceptor pairs, a saturating fraction E = k·c_A/(1+k·c_A) of donor energy is

transferred: the donor is quenched and the acceptor sensitised by E·absorbed_donor·Φ_A — a

product/saturating coupling that encodes dye co-localisation in the band ratio.

Fixed-pattern clutter. Many independent class-irrelevant spatial fields, each imprinted on a

random band subset with a random per-band gain — modelling real illumination/detector artefacts.

Clutter is high-variance but class-irrelevant; it is the dominant real-instrument confound (Figure 4).

Figure 3. Reabsorption reshapes the emission band. A bright pixel's emission spectrum at ex470 with and without reabsorption (strength 4): the blue edge is suppressed, producing an apparent red-shift — a concentration-dependent, nonlinear reshaping that relocates discriminative information into band shape.

Figure 4. Fixed-pattern clutter. Left/middle: the same band before and after injecting 36 class-irrelevant low-rank spatial clutter modes (amplitude 3.3). Right: sorted per-band variance — clutter raises the variance of many bands without adding any class information, which is exactly why variance/reconstruction-based selection collapses to chance.

## 2.7 The regimes

By dialing nuisance amplitude, scatter, photon/read noise, clutter, reabsorption and FRET, the generator

spans: clean (variance ≈ informativeness), realistic (strong nuisances + scatter + noise),

clutter (fixed-pattern instrument clutter), reabsorption / FRET (nonlinear), and spatial

(class carried by spatial texture). Exact parameters per figure/experiment are in the Appendix and in

paper/figures/params.json.

# 3. Methods

## 3.1 The AE + perturbation selector (and its convolutional variant)

A per-pixel (or spatial-convolutional) autoencoder is trained to reconstruct the standardised cube. Each

latent dimension is perturbed; the per-band reconstruction sensitivity is accumulated and normalised to a

per-band influence; the top-k bands (with a diversity constraint) are selected. The published CAE used

per-excitation 3-D convolution branches with an emission-axis average-pool ("band-collapse").

## 3.2 Baselines and references

random (the essential control), variance, pca_load (sum of absolute PCA loadings — the strongest

classical *blind* method), laplacian score; supervised references mutual_info (per-band) and

RF-importance / mRMR; the labels-using oracle; and full (all bands — the ceiling).

## 3.3 Evaluation protocol

Selection quality is downstream classification (a panel: KNN, RandomForest, MLP, logistic regression),

cross-validated or few-shot, reporting macro-F1 (and balanced accuracy, MCC, Cohen's κ). The decisive

honesty controls are (a) the random baseline — a method that cannot beat random adds nothing — and

(b) full-data — the ceiling a selector is trying to approach. For spatial discrimination we use

spatial-block cross-validation to remove spatial-autocorrelation leakage.

# 4. Experiments and results

## 4.1 The published CAE: four bugs and an architectural flaw

Run on modern tooling and evaluated honestly, the published CAE selected at chance (macro-F1 ≈ 0.33). We

found and fixed four real bugs in the trainer (a torch-compat crash, an UnboundLocalError on NaN

loss, a 0×NaN sparsity term that silently poisoned every non-sigmoid run, and an LR-collapse default),

corrected a misleading per-band-R² metric (off-peak noise bands dominate it; the honest metric is

signal-band correlation), and localised the residual failure to the emission-axis band-collapse,

which averages away exactly the multi-fluorophore structure. Removing the collapse took selection from

chance to ≈ 0.41 (≈ 85% of the labels-using oracle).

## 4.2 Rounds 1–2: the decision map

A comprehensive, significance-tested battery (5 metric families × 8 selectors × multiple regimes ×

seeds, with the random baseline) established the core results, summarised in Figure 5.

Figure 5. Headline results. Left: beat-random across five noise levels (k=24) — blind PCA/AE beat the random 95th-percentile ONLY at clean; under any clutter they collapse to random, and only supervised mutual-information beats random. Middle: compression curve — AE ≈ PCA at every budget (both reach ~95% of full at k≈64), supervised mutInfo compresses ~2.7× better, and random is competitive. Right: the spatial-texture regime under honest block-CV — per-pixel selectors (variance, mutInfo) are the WORST (blind by construction), while spatial methods and the oracle succeed; only a spatial model can find the discriminative bands.

Findings (all CI/seed-checked): blind selection never beats full-data (it only ties at very few

labels); supervised selection beats full only at clutter + ≲10 labels/class; AE ≈ PCA everywhere

(PCA better on clean, ties under clutter); and the AE's only significant edge — on nonlinear reabsorption

under one metric — did not reproduce at three seeds. The biggest single result: **under realistic

clutter, blind selection (AE and PCA) does not beat random band selection** — only label-aware selection

does. The mechanism is that clutter is high-variance but class-irrelevant, so every

variance/reconstruction criterion ranks clutter bands ≈ randomly with respect to class.

## 4.3 Round 3: a mechanism-driven mitigation campaign

We attacked each failure at its cause, with full model+data freedom: a **contrastive clutter-invariant

encoder (failed — instance discrimination rewards intrinsic clutter); a supervised AE + perturbation**

(captures the FRET/XOR interaction marginal MI misses, but margins are within noise and it never beats

full); AE-as-denoiser (helps +0.13 on clean — but it is *linear* low-rank denoising, matched or

beaten by PCA-reconstruction); and a sparse-signal regime (which only re-confirmed the wall: a

discriminative band is either bright → variance finds it, or dim → unextractable).

## 4.4 The one structural win: spatial discrimination

When class is carried by spatial texture (identical per-pixel marginals across classes), per-pixel

band selection is provably blind (Figure 5, right): under honest spatial-block CV the per-pixel

selectors are the *worst* (below random), while a spatial model + the right bands reach the oracle. This

is the one regime where the convolutional/spatial approach is structurally necessary — vindicating

the original spatial-CAE intuition, for spatial classification, not per-pixel spectral band selection.

A supervised spatial CNN selector beats the blind per-pixel method but still does not reliably *identify*

the discriminative bands — selection itself is intrinsically hard.

# 5. Discussion

Three mechanisms explain every negative result. (1) Selection ≠ classification: the AE's nonlinearity

helps a *classifier*, not the *choice* of bands; a nonlinear downstream model recovers structure (e.g.

XOR) from variance-prominent bands that simple methods already select. (2) Unsupervised ≡ variance:

reconstruction/variance/PCA-loadings all rank by variance, which under clutter or nuisances is decoupled

from class — so they perform like random. (3) Extractable ⇔ variance-prominent: a band strong enough

to be useful is high-variance (simple methods find it); a band that is not variance-prominent is too weak

to extract — leaving no gap for a learned selector to fill. The lone exception is genuinely *spatial*

information, invisible to per-pixel methods, where convolution is required.

# 6. Conclusion

SpectraForge is an honest, physics-grounded benchmark. On it, the autoencoder + latent-perturbation idea

— after thorough debugging — is a sound but not distinctive band selector: it matches PCA, fails to

beat a random control under realistic clutter, and never beats the full spectrum; selection's real value

is compression and, where it matters, it needs labels. The convolutional model's genuine, provable value

is as a spatial classifier where per-pixel methods fail. We release the generator, the full

experiment suite, and this appraisal so the result is reproducible and the scope is unambiguous.

# 7. References

J. R. Lakowicz, *Principles of Fluorescence Spectroscopy*, 3rd ed., Springer, 2006.

M. Kasha, "Characterization of electronic transitions in complex molecules," *Discuss. Faraday Soc.*

9, 14–19 (1950).

B. Valeur & M. N. Berberan-Santos, *Molecular Fluorescence*, 2nd ed., Wiley-VCH, 2012.

R. Bro, "PARAFAC: Tutorial and applications," *Chemom. Intell. Lab. Syst.* 38, 149–171 (1997).

A. Jabłoński, "Efficiency of anti-Stokes fluorescence in dyes," *Nature* 131, 839–840 (1933).

G. E. Hinton & R. R. Salakhutdinov, "Reducing the dimensionality of data with neural networks,"

*Science* 313, 504–507 (2006).

I. T. Jolliffe, *Principal Component Analysis*, 2nd ed., Springer, 2002.

T. Chen et al., "A simple framework for contrastive learning of visual representations," *ICML* 2020.

# Appendix A. Exact configurations

Every figure is generated by reports/paper_figures.py, which re-runs the simulations and writes the

exact parameters to paper/figures/params.json. Key generator parameters: emission 420–700 nm

(2 or 5 nm step → 141 or 57 bands), excitations {405, 470, 488, 506} nm, scene 64×64; realistic regime

nuisance_amp=1.2, photon_scale=2000, read_sigma=0.005; clutter modes=36, amp=3.3; reabsorption

strength=4; FRET k-dependent. The headline numbers in Figure 5 are produced by beat_random.py,

compression_curve.py, and spatial_regime.py; the full campaign log is docs/spectraforge/OVERNIGHT-LOG.md.